MEGA DRIVE EXTREME EDITION IV

USER MANUAL
REV 03

| Subscribe |  | @itnproducoes |
| --- | --- | --- |

MEGA DRIVE EXTREME EDITION IV USER MANUAL

With this version of Mega Drive you can:

→  Switch regions without hardware modification, with automatic PAL/NTSC switching via the reset button or the controller.

→ Reset function during gameplay using the controller.

→ Switchable overclocking during gameplay using the controller.

→ Automatic TMSS bypass.

→ Switch to 60Hz in EU/PAL games.

→ Pause and reset in Master System mode using the controller.

→ Use monochrome or color display (optional).

How to use Mega Drive mode:

→ To reset during gameplay, hold A+B+C+Start for 1 second.

→ To enable or disable overclock*, hold A+Start for 1 second. The LED will blink while overclock is active. Repeat the procedure to disable it. Example of games** where the difference is noticeable in overclock mode: Gunstar Heroes and Hard Drivin.

* Use overclock with caution, as this mode pushes the Mega Drive processor to its limits.

** Some games may show glitches in overclock mode because they were not designed for it.

→ To change the region*, using the reset button: press and hold it for 3 seconds. Using the controller: hold B+C+Start for 1 second.

Led Green → USA/NTSC region;

Led Red → JP/NTSC region;

Led Amber → EU/PAL50

Led Purple-→ EU/PAL60.

→ There are 4 possible LED brightness levels; to change them, press UP+Start for 1 second.

* The last selected region will be saved until changed again, even if the console is turned off.

→  To enable 60Hz* in EU/PAL games: with the console set to EU/PAL** and the game already running, hold A+B+Start for 1 second. To return to 50Hz, hold A+B+Start again or reset the console.

* Some European games are optimized for 50Hz; switching them to 60Hz may cause issues such as faster gameplay or desynchronized audio.

→ Full reset (same as turning the console off and on): hold A+C+Start for 3 seconds.

→ Standby mode, press UP+Start+A for 2 seconds. To return, press the reset button on the remote.

→ Disable or enable startup beep sounds and functions. Press UP+start+B for 2 seconds.
*When you turn the console off and on again, the last saved option (sound enabled or disabled) will be restored.

→ HOT: All functions can be accessed via the menu (of course, a connected display is required). Hold UP+START+C for 1 second. Navigate the menu with A and B. Change the selected option with C and exit the menu with START.

How to use Master System mode:

→ To pause the game using the controller, hold UP+B+C for 2 seconds.

→ To reset the game using the controller, hold UP+C for 5 seconds.

→ In Master System mode, you can change the region using the reset button by pressing and holding it for 3 seconds.

Using the display (optional):

WARNING: Never connect the display while the Mega Drive is powered on.

→ The display must be connected to the rear EXT port of the console while it is turned off.

→ In Mega Drive mode, press C+Start for 1 second to switch between logos. Continue pressing to return to the default information screen.

For consoles equipped with the BlueRetro module:

→  When powering on, hold UP for 1 second on the Mega Drive controller to enable or disable Bluetooth.

* When the console is turned off and on, the last saved option (module enabled or disabled) will be restored.

→ To enable or disable the Bluetooth module while the console is on, press and hold B+Start for 2 seconds.

→ When the module is activated, the LED will blink. (A little slower than when it is overclocked.)

** The BlueRetro module is not compatible with Master System mode.

Special thanks to DUSTINODELLOFFICIAL and the OPEN HEART project.

| Subscribe |  | @itnproducoes |
| --- | --- | --- |